# Packing list for Sampling Equipment

This list focuses especially on the items that, by experience, could be easily forgotten to pack. It is neither meant to be exhaustive; neither may all items be necessary in your specific use case.

- Beeswax (to cover drill holes, if allowed/desired by the institution)

- Brush (for cleaning)

- Cardboard paper, glossy black paper recommended for sampling of metal objects (to collect sample material that flew off the weighing paper)

- Centrifuge tubes, size according to sampling medium. Don't use glass vials for metal chips or curls; they will cling to the vials because they are statically charged (to store and transported sampled material)

- Cling film (to cover surfaces for sampling)

- Ethanol (for cleaning)

- Gloves, single-use gloves or (white) cotton gloves (for general handling of objects)

- Magnifier, handheld or as headband, combination with lighting is useful

- Paper wipes (for cleaning)

- Permanent marker, waterproof and abrasion-resistant (to label vials and/or bags)

- Pointed tweezers

- Power adapter

- Power extension cord

- Precision pocket balance (to weigh objects before and after sampling)

- Sample bags, plastic, sealable (to store e.g. centrifuge tubes with sampled material)

- Sample labels for institutions, see example in the appendix (to document which institution carried out the sampling and the subsequent analyses)

- Scale, combination with colour scale/colour checker is useful (for photography)

- Scalpel

- Scissors

- Self-adhesive labels

- Spatula

- Tape

- Voltage converter

- Weighing paper (to collect material during sampling by drilling and tweezing, clipping, sawing etc.)